Refernce Pulse Generation And Detection Code Flash On SGLD: 


// ============================================================

// SG/LD V1 - SIMULATED METER PULSE GENERATOR

//

// GPIO27 = EXISTING GREEN LED + EXTERNAL PULSE OUTPUT

//

// Initial reading:

//     10

//

// READY:

//

//     Pulse 1 -> 11

//     Pulse 2 -> 12

//     Pulse 3 -> 13

//     Pulse 4 -> 14

//

// GPIO27:

//     HIGH = Pulse + Green LED ON

//     LOW  = Pulse OFF + Green LED OFF

// ============================================================

#define PULSE_PIN 27

// ============================================================

// METER VARIABLES

// ============================================================

uint32_t initialReading = 0;

uint32_t currentReading = 0;

bool running = false;

// ============================================================

// PULSE TIMING

// ============================================================

// Time from one pulse to the next

const unsigned long PULSE_INTERVAL = 2000;

// HIGH duration of each pulse

// Increased from 100 ms to 1000 ms

const unsigned long PULSE_WIDTH = 1000;

unsigned long lastPulseTime = 0;

// ============================================================

// SETUP

// ============================================================

void setup()

{

Serial.begin(115200);

delay(500);

pinMode(PULSE_PIN, OUTPUT);

digitalWrite(PULSE_PIN, LOW);

Serial.println();

Serial.println("========================================");

Serial.println("       SG/LD METER PULSE GENERATOR");

Serial.println("========================================");

Serial.println();

Serial.println("GPIO27 = GREEN LED + PULSE OUTPUT");

Serial.println();

Serial.println("Enter initial reading.");

Serial.println("Example:");

Serial.println("10");

Serial.println();

Serial.println("Then enter:");

Serial.println("READY");

Serial.println();

Serial.println("Commands:");

Serial.println("10       -> Set reading");

Serial.println("READY    -> Start");

Serial.println("STOP     -> Stop");

Serial.println("STATUS   -> Status");

Serial.println();

Serial.println("Waiting for initial reading...");

}

// ============================================================

// LOOP

// ============================================================

void loop()

{

// ========================================================

// SERIAL INPUT

// ========================================================

if (Serial.available())

{

String cmd = Serial.readStringUntil('\n');

cmd.trim();

cmd.toUpperCase();

// ====================================================

// NUMBER CHECK

// ====================================================

bool numberOnly = true;

if (cmd.length() == 0)

{

numberOnly = false;

}

for (int i = 0; i < cmd.length(); i++)

{

if (!isDigit(cmd[i]))

{

numberOnly = false;

break;

}

}

// ====================================================

// INITIAL READING

// ====================================================

if (numberOnly)

{

initialReading = cmd.toInt();

currentReading = initialReading;

running = false;

digitalWrite(PULSE_PIN, LOW);

Serial.println();

Serial.println("----------------------------------------");

Serial.print("Initial Reading = ");

Serial.println(initialReading);

Serial.print("Current Reading = ");

Serial.println(currentReading);

Serial.println("----------------------------------------");

Serial.println();

Serial.println("Enter READY");

Serial.println();

}

// ====================================================

// READY

// ====================================================

else if (cmd == "READY")

{

if (initialReading == 0)

{

Serial.println();

Serial.println("ERROR!");

Serial.println("Set initial reading first.");

Serial.println("Example: 10");

Serial.println();

}

else

{

running = true;

lastPulseTime = millis();

Serial.println();

Serial.println("========================================");

Serial.println("              READY");

Serial.println("========================================");

Serial.print("Starting Reading = ");

Serial.println(currentReading);

Serial.println();

Serial.println("Pulse generation started.");

Serial.println();

}

}

// ====================================================

// STOP

// ====================================================

else if (cmd == "STOP")

{

running = false;

digitalWrite(PULSE_PIN, LOW);

Serial.println();

Serial.println("PULSE GENERATION STOPPED");

Serial.println();

}

// ====================================================

// STATUS

// ====================================================

else if (cmd == "STATUS")

{

Serial.println();

Serial.println("------------- STATUS -------------");

Serial.print("Initial Reading = ");

Serial.println(initialReading);

Serial.print("Current Reading = ");

Serial.println(currentReading);

Serial.print("Running = ");

if (running)

{

Serial.println("YES");

}

else

{

Serial.println("NO");

}

Serial.println("----------------------------------");

Serial.println();

}

// ====================================================

// UNKNOWN COMMAND

// ====================================================

else

{

Serial.println();

Serial.println("Unknown command.");

Serial.println("Use:");

Serial.println("10");

Serial.println("READY");

Serial.println("STOP");

Serial.println("STATUS");

Serial.println();

}

}

// ========================================================

// GENERATE PULSE

// ========================================================

if (running)

{

if (millis() - lastPulseTime >= PULSE_INTERVAL)

{

lastPulseTime = millis();

generatePulse();

}

}

}

// ============================================================

// GENERATE PULSE

// ============================================================

void generatePulse()

{

// --------------------------------------------------------

// Increase reading

// --------------------------------------------------------

currentReading++;

Serial.println();

Serial.println("========================================");

Serial.println("             NEW PULSE");

Serial.println("========================================");

Serial.print("Meter Reading = ");

Serial.println(currentReading);

Serial.println("GPIO27 = HIGH");

Serial.println("GREEN LED = ON");

// ========================================================

// HIGH

//

// Same GPIO27:

//

// 1. Green LED ON

// 2. Pulse signal HIGH

// 3. Receiver ESP32 detects rising edge

// ========================================================

digitalWrite(PULSE_PIN, HIGH);

delay(PULSE_WIDTH);

// ========================================================

// LOW

// ========================================================

digitalWrite(PULSE_PIN, LOW);

Serial.println("GPIO27 = LOW");

Serial.println("GREEN LED = OFF");

Serial.println("========================================");

}

Receiver on ESP32 code :


// ============================================================

// ESP32 DEVKIT V1

// BLE + SERIAL MONITOR + PULSE METER RECEIVER

//

// GPIO27 = PULSE INPUT

//

// SG/LD BOARD GPIO27

//        |

//        | PULSE

//        v

// ESP32 DEVKIT GPIO27

//

// BLE DEVICE NAME:

// ESP32_PULSE_METER

//

// COMMANDS:

// 10       -> Set initial reading

// READY    -> Start pulse monitoring

// STOP     -> Stop monitoring

// RESET    -> Reset pulse count

// STATUS   -> Show status

//

// Example:

//

// 10

// READY

//

// Pulse 1 -> Reading 11

// Pulse 2 -> Reading 12

// Pulse 3 -> Reading 13

// ============================================================

#include <BLEDevice.h>

#include <BLEServer.h>

#include <BLEUtils.h>

#include <BLE2902.h>

// ============================================================

// GPIO

// ============================================================

#define PULSE_INPUT_PIN 27

// ============================================================

// BLE DEVICE NAME

// ============================================================

#define BLE_DEVICE_NAME "ESP32_PULSE_METER"

// ============================================================

// NORDIC UART SERVICE UUID

// ============================================================

#define SERVICE_UUID \

"6E400001-B5A3-F393-E0A9-E50E24DCCA9E"

#define RX_CHARACTERISTIC_UUID \

"6E400002-B5A3-F393-E0A9-E50E24DCCA9E"

#define TX_CHARACTERISTIC_UUID \

"6E400003-B5A3-F393-E0A9-E50E24DCCA9E"

// ============================================================

// BLE OBJECTS

// ============================================================

BLEServer *pServer = nullptr;

BLECharacteristic *pTxCharacteristic = nullptr;

BLECharacteristic *pRxCharacteristic = nullptr;

bool deviceConnected = false;

// ============================================================

// METER VARIABLES

// ============================================================

volatile uint32_t pulseCount = 0;

volatile unsigned long lastPulseTime = 0;

uint32_t initialReading = 0;

uint32_t currentReading = 0;

bool monitoring = false;

// ============================================================

// PULSE FILTER

// ============================================================

const unsigned long MIN_PULSE_INTERVAL_MS = 50;

// ============================================================

// FUNCTION PROTOTYPES

//

// IMPORTANT:

// These declarations solve the compilation errors:

// 'sendBLE' was not declared

// 'processCommand' was not declared

// ============================================================

void sendBLE(String message);

void sendMessage(String message);

void processCommand(String command);

void generateStatus();

// ============================================================

// PULSE INTERRUPT

// ============================================================

void IRAM_ATTR pulseISR()

{

unsigned long now = millis();

if ((now - lastPulseTime) >= MIN_PULSE_INTERVAL_MS)

{

pulseCount++;

lastPulseTime = now;

}

}

// ============================================================

// BLE SERVER CALLBACKS

// ============================================================

class ServerCallbacks : public BLEServerCallbacks

{

void onConnect(BLEServer *server)

{

deviceConnected = true;

Serial.println();

Serial.println("================================");

Serial.println("BLE CONNECTED");

Serial.println("================================");

Serial.println();

// Send only after BLE connection exists

sendBLE("BLE CONNECTED");

}

void onDisconnect(BLEServer *server)

{

deviceConnected = false;

Serial.println();

Serial.println("BLE DISCONNECTED");

Serial.println();

// Restart advertising

BLEDevice::startAdvertising();

Serial.println("BLE advertising restarted.");

}

};

// ============================================================

// BLE RX CALLBACK

//

// PHONE -> ESP32

// ============================================================

class RXCallbacks : public BLECharacteristicCallbacks

{

void onWrite(BLECharacteristic *characteristic)

{

String command = characteristic->getValue().c_str();

command.trim();

command.toUpperCase();

if (command.length() > 0)

{

Serial.print("BLE COMMAND: ");

Serial.println(command);

processCommand(command);

}

}

};

// ============================================================

// SEND BLE MESSAGE

//

// ESP32 -> PHONE

// ============================================================

void sendBLE(String message)

{

if (deviceConnected && pTxCharacteristic != nullptr)

{

pTxCharacteristic->setValue(message.c_str());

pTxCharacteristic->notify();

}

}

// ============================================================

// SEND TO SERIAL + BLE

// ============================================================

void sendMessage(String message)

{

Serial.println(message);

sendBLE(message);

}

// ============================================================

// PROCESS COMMAND

// ============================================================

void processCommand(String command)

{

command.trim();

command.toUpperCase();

// ========================================================

// CHECK WHETHER COMMAND IS A NUMBER

//

// Example:

// 10

// 25

// 100

// ========================================================

bool numberOnly = true;

if (command.length() == 0)

{

numberOnly = false;

}

for (int i = 0; i < command.length(); i++)

{

if (!isDigit(command[i]))

{

numberOnly = false;

break;

}

}

// ========================================================

// INITIAL READING

// ========================================================

if (numberOnly)

{

uint32_t value = command.toInt();

noInterrupts();

initialReading = value;

pulseCount = 0;

currentReading = initialReading;

lastPulseTime = 0;

interrupts();

monitoring = false;

Serial.println();

Serial.println("--------------------------------");

Serial.print("Initial Reading = ");

Serial.println(initialReading);

Serial.print("Current Reading = ");

Serial.println(currentReading);

Serial.println("--------------------------------");

Serial.println();

Serial.println("Enter READY.");

Serial.println();

sendBLE(

"Initial Reading = " +

String(initialReading)

);

sendBLE(

"Current Reading = " +

String(currentReading)

);

sendBLE("Enter READY");

return;

}

// ========================================================

// READY

// ========================================================

if (command == "READY")

{

if (initialReading == 0)

{

sendMessage(

"ERROR: Set initial reading first."

);

return;

}

noInterrupts();

pulseCount = 0;

currentReading = initialReading;

lastPulseTime = millis();

interrupts();

monitoring = true;

sendMessage("================================");

sendMessage("READY");

sendMessage(

"Starting Reading = " +

String(currentReading)

);

sendMessage("Waiting for pulses...");

sendMessage("================================");

return;

}

// ========================================================

// STOP

// ========================================================

if (command == "STOP")

{

monitoring = false;

sendMessage("PULSE MONITORING STOPPED.");

return;

}

// ========================================================

// RESET

// ========================================================

if (command == "RESET")

{

noInterrupts();

pulseCount = 0;

currentReading = initialReading;

interrupts();

sendMessage("PULSE COUNTER RESET.");

sendMessage(

"Current Reading = " +

String(currentReading)

);

return;

}

// ========================================================

// STATUS

// ========================================================

if (command == "STATUS")

{

generateStatus();

return;

}

// ========================================================

// UNKNOWN COMMAND

// ========================================================

sendMessage("Unknown command.");

sendMessage("Available commands:");

sendMessage("10");

sendMessage("READY");

sendMessage("STOP");

sendMessage("RESET");

sendMessage("STATUS");

}

// ============================================================

// STATUS

// ============================================================

void generateStatus()

{

uint32_t count;

noInterrupts();

count = pulseCount;

interrupts();

currentReading =

initialReading + count;

sendMessage("------------- STATUS -------------");

sendMessage(

"Initial Reading = " +

String(initialReading)

);

sendMessage(

"Pulse Count = " +

String(count)

);

sendMessage(

"Current Reading = " +

String(currentReading)

);

if (monitoring)

{

sendMessage("Monitoring = RUNNING");

}

else

{

sendMessage("Monitoring = STOPPED");

}

sendMessage("----------------------------------");

}

// ============================================================

// SETUP

// ============================================================

void setup()

{

// ========================================================

// SERIAL

// ========================================================

Serial.begin(115200);

delay(1000);

// ========================================================

// PULSE INPUT

// ========================================================

pinMode(PULSE_INPUT_PIN, INPUT);

attachInterrupt(

digitalPinToInterrupt(PULSE_INPUT_PIN),

pulseISR,

RISING

);

// ========================================================

// BLE INITIALIZATION

// ========================================================

BLEDevice::init(BLE_DEVICE_NAME);

// ========================================================

// CREATE BLE SERVER

// ========================================================

pServer =

BLEDevice::createServer();

pServer->setCallbacks(

new ServerCallbacks()

);

// ========================================================

// CREATE UART SERVICE

// ========================================================

BLEService *pService =

pServer->createService(

SERVICE_UUID

);

// ========================================================

// TX

//

// ESP32 -> PHONE

//

// NOTIFY

// ========================================================

pTxCharacteristic =

pService->createCharacteristic(

TX_CHARACTERISTIC_UUID,

BLECharacteristic::PROPERTY_NOTIFY

);

pTxCharacteristic->addDescriptor(

new BLE2902()

);

// ========================================================

// RX

//

// PHONE -> ESP32

//

// WRITE

// ========================================================

pRxCharacteristic =

pService->createCharacteristic(

RX_CHARACTERISTIC_UUID,

BLECharacteristic::PROPERTY_WRITE |

BLECharacteristic::PROPERTY_WRITE_NR

);

pRxCharacteristic->setCallbacks(

new RXCallbacks()

);

// ========================================================

// START SERVICE

// ========================================================

pService->start();

// ========================================================

// START BLE ADVERTISING

// ========================================================

BLEAdvertising *advertising =

BLEDevice::getAdvertising();

advertising->addServiceUUID(

SERVICE_UUID

);

advertising->setScanResponse(true);

advertising->setMinPreferred(0x06);

advertising->setMinPreferred(0x12);

BLEDevice::startAdvertising();

// ========================================================

// STARTUP MESSAGE

// ========================================================

Serial.println();

Serial.println();

Serial.println("========================================");

Serial.println("       ESP32 PULSE METER + BLE");

Serial.println("========================================");

Serial.println();

Serial.print("BLE Name: ");

Serial.println(BLE_DEVICE_NAME);

Serial.println();

Serial.print("Pulse Input: GPIO");

Serial.println(PULSE_INPUT_PIN);

Serial.println();

Serial.println("BLE advertising started.");

Serial.println();

Serial.println("Commands:");

Serial.println("10       -> Set initial reading");

Serial.println("READY    -> Start monitoring");

Serial.println("STOP     -> Stop monitoring");

Serial.println("RESET    -> Reset pulse count");

Serial.println("STATUS   -> Show status");

Serial.println();

Serial.println("Waiting for initial reading...");

}

// ============================================================

// LOOP

// ============================================================

void loop()

{

// ========================================================

// SERIAL MONITOR COMMAND

// ========================================================

if (Serial.available())

{

String command =

Serial.readStringUntil('\n');

command.trim();

command.toUpperCase();

if (command.length() > 0)

{

Serial.print("SERIAL COMMAND: ");

Serial.println(command);

processCommand(command);

}

}

// ========================================================

// UPDATE READING

// ========================================================

if (monitoring)

{

uint32_t count;

noInterrupts();

count = pulseCount;

interrupts();

currentReading =

initialReading + count;

// ----------------------------------------------------

// Detect new pulse

// ----------------------------------------------------

static uint32_t previousCount = 0;

if (count != previousCount)

{

previousCount = count;

String message;

message =

"PULSE DETECTED | ";

message +=

"Pulse Count = ";

message +=

String(count);

message +=

" | Reading = ";

message +=

String(currentReading);

// Send to Serial Monitor

// AND BLE phone

sendMessage(message);

}

}

delay(5);

}